Assumptions:

- No fringing: flux goes straight from the air gap

- No flux leakage: flux goes through the cores uniformly

- Sinusoidal variation of flux: flux changes as a sin wave that has the peaks at 90 deg and 0 deg of the rotor which is 0.5 & 2.5 mm air gap

- İnfinitely permeable stator and rotor is assumed thus reluctance for these parts is zero

- No saturation is assumed

Reluctance computation

R = 2*lμ0A where l: air gap, A: Effective area, μ0=air permeability

A = 2πrθ360l  where θ is the angle that looks to the 15 mm width of the rotor and it is 77.36 deg

Then Rmin =2*lminμ0A=1,965*106  1/H

And

Rmax =2*lmaxμ0A=9,824*106  1/H

Inductance computation

L = N2R         ->          Lmax=3002Rmin=45,8 mH   ,    Lmin=3002Rmax=9.2 mH

Assuming a sinisoidal variation

L(θ) = L0+L1cos(2θ)  from L(0) = Lmax  &  L(90) = Lmin  the equation becomes

L(θ) = 27.5+18.3cos(2θ)

Torque computation

T = 12I2dL(θ)dθ

T(θ) = -0.114*sin(2θ)

Non linear magnetic circuits

As we assumed infinite permeable material with no saturation without this assumption non linearity should be accounted.

Rstator and Rrotor should be calculated. To calculate them a flux value can be assumed then BH curve is checked then reluctance calculated again accordingly. And an iterative solution is utilized.

Non homogeneous Flux

We assumed flux travelled uniformly in one single loop without this assumption flux leaks into the center and concentrates heavily at sharp inner corners.

To calculate one needs to build a Magnetic Equivalent Circuit network. There would be many interconnected reluctances and this network would look like a grşd of complex mesh of electrical resistors.

Then using matrix algebra the flux could be calculated.

Fringing flux Due to fringing flux the reluctance of the machine would decrease since the effective area inscreases. Due to reluctance decrease inductance increases. To calculate inductance with including fringing many models are suggested. For instance Kazimierczuk in his high frequency magnetic components book suggests using a factor f such that

Formula

Where a,b are the lengths of each side of the rectangular cross section of magnetic core.

Q4

As the reluctance torque direction does not depend on current sign an on/off DC current can be utilized to rotate the rotor in for example counter clockwise direction. There should be a DC current at positions where the torque is at counter clockwise direction. When the torque is in clockwise direction current should be 0 and we would expect rotor to rotate against the friction and come to a position where the torque is again counter clockwise.